TCSS 371A

Lab Project 2

Due 11/5/08 (Working Simulations – 1 per group- due 10/29)

Work in groups of 2 again.  If your group is different from Project 1, please let me know.

1) The second project will be to implement a computer memory:

- Memory: The address space, or addressability, is 4 words.  The word width is 8 bits.

- Memory Address Register: There will be a 2 bit Memory Address Register that holds

the memory address of the memory word to be read from or written into.

- Memory Data Register: There will be an 8 bit Memory Data Register that holds the

data to be written into a memory word, or to hold the data read from a memory word.

- R/W: There will be a control bit that specifies whether there will be a memory read (read from memory and place the result in the Memory Data Register) or a memory write (read from the Memory Data Register and place the result in the memory)

- Transfer: There will be a Transfer signal that will cause the read from memory or write to memory to occur when it transitions from a “0” to a “1”.

Design and test your memory.  Use any devices that are defined in the LogicWorks library.  I would try to keep the chip count as low as practical.

- Use registers to simulate the words of memory.

- Use a register to simulate the Memory Data Register.

- Use switches to simulate the Memory Address Register and the R/W bit.

- Choose the optimum size Decoders and MUX’s

- Use an S/R Flip Flop to simulate the Transfer signal.  Why is this a good choice?

2) Simulate and build a 2 word by 2 bit memory, implementing the storage devices with 7474 D Flip Flops and implementing the combinational logic using 2 input gates.  Include an MDR.

Your Individual Report should include:

Heading:

TCSS371 Lab Project 1                           Name                                                  Date submitted

Partner’s Name

1) Introduction – A paragraph providing an “overview” of the project suitable to describe the project to a novice.

2) Project Description – Brief, but coherent descriptions of what you did with a few schematics and timing windows to document the project.

3) Conclusion – A brief description of what you learned and what gave you difficulty.  Give appropriate credit to anyone who helped you.

4) References – List references used: books, papers, web pages.

Note: A report should be presented professionally in a clear and concise style, with enough detail to make it meaningful and comfortable to read and follow.