TCSS465 Embedded Real-Time System Programming

Lab Exorcise 3:

- Run the NetBurner Mod5282 Interrupts Application Note’s example program that services interrupts from the IRQ switch.  See below.

- Write a program that uses the IRQ switch in unique application of your choice.  For example, you might use the LED’s to simulate a game with a ball moving, do something with the displays, or do something that includes the A/D converter you are now familiar with.  Use some assembly language in your program, perhaps in the service routine.  Show your creativity.

NetBurner Mod5282 Interrupts Application Note’s example program:

#include "predef.h"

#include <stdio.h>

#include <ctype.h>

#include <startnet.h>

#include <autoupdate.h>

#include <dhcpclient.h>

#include <../mod5282/system/sim5282.h>

#include <cfinter.h>

/* An interrupt setup helper defined in bsp.c */

extern "C"

{

/* This function sets up the 5282 interrupt controller */

void SetIntc(int intc,long func, int vector, int level,int prio );

}

/* We are going to use a Semaphore to communicate between the IRQ1 pin ISR and the main application */

OS_SEM IrqPostSem;

/* Declare our interrupt procedure....

name: our_irq1_pin_isr

masking level (The value of the ColdFire SR during the interrupt:

use 0x2700 to mask all interrupts.

0x2500 to mask levels 1-5 etc...

0x2100 to mask level 1 */

INTERRUPT(out_irq1_pin_isr, 0x2100 )

{

/* WARNING WARNING WARNING

Only a very limited set of RTOS functions can be called from within an interrupt service routine.

Basically, only OS POST functions and LED functions should be used

No I/O (read, write or printf may be called), since they can block. */

sim.eport.epfr=0x02; /* Clear the interrupt edge 0 0 0 0 0 0 1 0 */

OSSemPost(&IrqPostSem);

}

extern "C"

{

void UserMain(void * pd);

void putdisp(unsigned short w);

}

/* Helper function to display a decimal number */

void PutDispDecimal(WORD val, BOOL blank_zero )

{

WORD w;

w = (val / 1000) * 0x1000;

if ((w == 0) && (blank_zero)) w = 0xF000;

w += ((val / 100)% 10) * 0x0100;

if (w == 0xF000) w = 0xFF00;

w += ((val / 10)% 10) * 0x0010;

if (w == 0xFF00) w = 0xFFF0;

w += ((val )% 10) * 0x0001;

putdisp(w);

}

void UserMain(void * pd)

{

InitializeStack();

if (EthernetIP==0)GetDHCPAddress();

OSChangePrio(MAIN_PRIO);

EnableAutoUpdate();

DWORD isr_count=0; /* Count how many times the switch was hit */

/* Initialize the semaphore we are using */

OSSemInit(&IrqPostSem,0);

/* First set up the Eport module to use IRQ1 as a falling-edge active IRQ pin

(See the 5282 UM chapter 11). Set the pin assignment register irq1 pin falling

edge sensitive*/

sim.eport.eppar=0x0008; /* 00 00 00 00 00 00 10 00 see table 11-13 in UM */

sim.eport.epddr=0x0; /* All edge port pins as inputs */

sim.eport.epier = 0x0002; /* Enable IRQ1 only 0 0 0 0 0 0 1 0 */

/* Now enable the actual interrupt controller. See users manual chapter 10 for

more information. We are going to use the BSP helper function declared above and

implemented in BSP.c */

SetIntc(0,/* The first interrupt controller */

(long)&out_irq1_pin_isr, /* Our interrupt function */

1, /* The vector number from the users manual table 10-13 */

1, /* Set this to priority 1 but any value from 1 to 6 would be valid.*/

1 /* The priority within the gross levels; see chapter 10, any value from 0 to 7 is ok */);

iprintf("Application started\n");

iprintf("Press the IRQ button on the development board.\n");

while (1)

{

OSSemPend(&IrqPostSem,0 /* Wait forever */);

PutDispDecimal(++isr_count,true);

iprintf("The interrupt Switch was hit %ld times\r\n", isr_count);

}

}