Lab exercise 4:                                                     Due date 5/23/06

- Run the NetBurner Mod5282 Interrupts Application Note’s example program that services interrupts from the IRQ switch.

- Write a program that uses the IRQ switch in a different way.  For example, you might use the LED’s to simulate a ball moving.  An interrupt generated by the IRQ switch could make it change direction, or ?   Use some assembly language in your program, perhaps in the service routine.

NetBurner Mod5282 Interrupts Application Note’s example program:

void UserMain(void * pd)

{

InitializeStack();

if (EthernetIP==0)GetDHCPAddress();

OSChangePrio(MAIN_PRIO);

EnableAutoUpdate();

DWORD isr_count=0; /* Count how many times the switch was hit */

/* Initialize the semaphore we are using */

OSSemInit(&IrqPostSem,0);

/* First set up the Eport module to use IRQ1 as a falling-edge active IRQ pin

(See the 5282 UM chapter 11).

Set the pin assignment register irq1 pin falling edge sensitive.

*/

sim.eport.eppar=0x0008;   /* 00 00 00 00 00 00 10 00 see table 11-3 in UM */

sim.eport.epddr=0x0;      /* All edge port pins as inputs */

sim.eport.epier = 0x0002; /* Enable IRQ1 only 0 0 0 0 0 0 1 0 */

/* Now enable the actual interrupt controller. See users manual chapter 10 for

more information. We are going to use the BSP helper function declared above

and implemented in BSP.c

*/

SetIntc(0, /* The first interrupt controller */

(long)&out_irq1_pin_isr, /* Our interrupt function */

1, /* The vector number from the users manual table 10-13 */

1, /* Set this to priority 1 but any value from 1 to 6 would

be valid.*/

1  /* The priority within the gross levels; see chapter 10, any

value from 0 to 7 is ok */

);

iprintf("Application started\n");

iprintf("Press the IRQ button on the development board.\n");

while (1)

{

OSSemPend(&IrqPostSem,0 /* Wait forever */);

PutDispDecimal(++isr_count,true);

iprintf("The interrupt Switch was hit %ld times\r\n", isr_count);

}

}

Lab exercise 5:                                                    Due Date 6/01/06

1)   Create an interrupt driven application in which two devices on the NetBurner Board generate interrupts.  They could be a timer, the A/D converter, and/or a Universal Asynchronous Receiver / Transmitter (UART).   Be creative.  Be prepared to demonstrate your design June 1st.