Lab exercises for 4/20/06:                                      Due date 4/28/06

- First, finish the Lab exercises for 4/13/06.  Don’t jump to the next task before finishing it.

- This week we want to exercise the A/D converter with a more typical input device.  We will use a thermistor with the  NetBurner to make a thermometer:

### Input to A/D converter

## Input to I  Inp

- Research thermistors.   Describe how they work and explain the relationship between their resistance and their temperature.   Normally thermistors are specified by 1) Nominal Resistance, 2) B Constant, and 3)Temperature Coefficient.  Explain the thermistor temperature vs resistance curve in terms of these parameters.

- We will be using a Jameco Electronics P/N 207036 thermistor.  Go to the Jameco web site and obtain the data sheets for it.  Show the voltage input versus temperature curve you expect to observe.

- Write a NetBurner program to test your “system.”

- Construct your thermistor circuit.  Use 5 volts for +V (from the NetBurner board – test the voltage using the scope to make certain you are using the correct power pin – same for gnd) and use a 10K resistor for R.

- Test your thermister circuit, first without the NetBurner and then with it.  Apply several known temperatures to test it (use a lab thermometer).  How do the values compare to those expected?

- Modify your program to directly read out the temperature of your thermistor circuit.

- Determine the accuracy of your thermometer.