Lab exercises for 4/6/06:

Last session you presumably:

- Followed the start-up instructions in the start-up documents:

- NNDK Programming Guide (pages 1-20)

- Development C++ Quick Start Guide

- The NetBurner Cook Book – Getting Started (in the NNDK Users Manual)

  - Played with the Factory Demo Program (Including the HTML page)

  - Wrote some simple programs of your own and played with them\

  - Investigated some example programs in the Nburn folder in the C drive.

This session we want to continue getting familiar with the NetBurner:

- Write a program that uses the 8 LED’s:

You can use the method:   void  putleds ( unsigned int i);

- Write a program that uses the 8 Dip Switches:

You can use the method:  unsigned int  getdipsw ();

- Write a program that uses the 4  7-segment LED’s:

You can use the method:  void putdisp ( unsigned int i);

{the display expects a 4 digit Binary-Coded-Decimal number}

- Write a program that uses the A/D converter as an output device:

See the MCF5282 User’s Manual (There is a link in the Platform Hardware document) Sections 27.4 & 27.6

Set the   sim.qadc.ddrqa   Port A Data Direction Register for pins  0,1,3, & 4 to be digital outputs

Set the outputs with  sim.qadc.portqa  Port A Data Register

Use the Oscilloscope to observe the output (s).  (See the Schematics in the Platform Hardware document)

{ PLEASE BE VERY CAREFUL using the scope probes!  We don’t want to sacrifice any NetBurners}

- Write up you programs with documentation to share.  Give team members credit for their contributions and acknowledge appropriately help received from others.