NetBurner Ethernet Initialization:

Launch the MTTTY Serial Terminal Program from the Desktop\ECS Lab\Netburner NNDK  folder

(Communication parameters should be set to:  COM2, 115200, None, 8, 1)

Start the NetBurner Monitor:

Click “connect”  => A prompt will appear

Press the “Reset” button on the NetBurner Board  => “Waiting 2sec to start   ‘A’ to abort” will appear

Enter  “A”  within 2 sec  =>  “nb>” and a prompt will appear    (Press reset again if you took to long)

Type Help  =>  Lists help menu

Choose Setup & check/enter IP parameters

IP Address = 10.1.1.x    x = number of your Host/NetBurner station  (1 <= x  <= 16 )

IP Mask = 255.255.255.0

IP Gateway = IP Address of your host is displayed on the Desktop

(or open DOS window enter IPCONFIG)

Save the new IP information,   and exit

Load the Factory Demo Program:

Enter “FLA”  =>  “Begin Down Load now …  ”   will appear

Choose  Transfer ,  Send File (text) ,  C:Nburn\MOD5282\original\Mod 5282FactoryDemo_APP.S19

=> the file will transfer, and the Application will run.  Don’t abort the application.

Create the network routing:

Open the  ECS Lab  folder.  Double click “set up route”.   The DOS window will flash by.

From the Factory Demo Application, ping your host and your NetBurner to ensure the Ethernet connection is working.

Make a folder for your applications:

The NetBurner’s default folder for projects is C:\Nburn\examples.  Since you don’t have write privileges on C:, create a folder called examples on your H: drive.   Open a DOS window and type the command  C:\Nburn\ setenv.  Then redirect project defaults from C:\Nburn\examples to H:\examples: