Oscilloscope Settings:

Scope layout:

-------------------------------------------------------------------

Horizontal								Run Control

Measure	    Waveform

File								     Trigger

Analog								      Digital

-------------------------------------------------------------------

Initial Set-Up:

# Digital

All lights off

# Analog

Lights 1 & 2 on

# Horizontal

Main (not delayed!)

# Run Control

RUN/STOP on

# Trigger  - Mode Coupling

Auto      DC Coupling	    No Noise Rej    No HF Rej    60ns holdoff    Edge (up)    1

Edge Light on

Suggested Nominal Settings:

## Analog

Set Channel 1 & 2 to 2.00 volts/div (use top knobs – see top left of screen)

Set Channel 1 line at 3 div down (use bottom knob)

Set Channel 2 line at 5 div down or 1 up (use bottom knob)

### Trigger

Set trigger level at 2.5 volts (use knob – see top right corner of screen)

## Horizontal

Set sweep at about 50 microsec (use left knob - see top middle of screen)

Set delay at 0.0S (use right knob - see top middle of screen)

## Run Control

Set to Run/Stop (green light)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Top of screen parameters:

1  2.00V/     2  2.00V/                            0.0S     50 microsec/               Auto(blinking)        UP      1      2.50V