| # Standard Operating Procedure for Silanaes (e.g. dicholorodimethylsilane) |  |
| --- | --- |
| 1. Health hazards | Highly flammable, toxic if swallowed or inhaled, corrosive to metals and skin, serious eye damage |
| 2. Training/Approval | Hazardous chemical training and reading of this SOP is required before working with silanes. |
| 3. Personal Protective Equipment (PPE) | Nitrile gloves and safety googles. |
| 4. Environmental / Ventilation Controls | Opening and aliquoting of silanes should be performed in a fume hood. |
| 5. Special Handling Procedures & Storage Requirements | Handling: Silanes should only be handled in a fume hood while wearing proper safety attire. To minimize hazards, only use small volumes (< 1 mL). Storage: Store in a sealed, dry container (i.e. containing drierite) underneath the hood. |
| 6. Spill and Accident Procedures | - Small spills in the fume hood will evaporate on their own - Large spills should be cleaned using Kim Wipes. The Kim wipes should be disposed of through EH&S Exposure: - In case of contact on skin, immediately remove any affected clothing and rinse with water. Consult with Employee Health Center, 685-1026. - For eye exposure, flush with water for at least 15 minutes. Immediately consult with Employee Health Center, 685-1026. After hours, go to the UWMC or HMC Emergency Department. - If inhaled, go to fresh air and stay at rest in a position comfortable for breathing. - If swallowed, rinse mouth but do not induce vomiting. Immediately call poison control. - Report incident to supervisor. Supervisor reports the accident/injury on UW OARS. |
| 7. Waste Disposal | Silanization produces minimal waste, as most leaves as a vapor. For liquid waste, consult with EH&S Environmental Programs at 616-2848. For pick-up, complete a Chemical Collection Request Form and mail to Box 354110 or FAX to 206-685-2915 |
| 8. Precautions for Animal Use | Do not use with animals. |
| 9. Decontamination | Dessicators used with silanes should be clearly labeled and left uncleaned. Surfaces accidently exposed should be rinsed with toluene three times, then ethanol three times. |
| 10. Designated Area | Silanes should be used only in the fume hood. |
| Name: Sarah Keller Title: PI |  |
| Signature: Date: July 21, 2014 |  |

EH&S Research and Biological Safety Office, 206-221-7770